## Aether - Trajectory

# Mission Analysis details

TBD: To Be determined

# Before launch

## 1.1 Initialisation

Enter: When the system is turned on OR when the system reset

Actions: Initialise everything: clocks, variables, registry, …

Exit: Once everything is initialised AND the umbilical is plugged -> Go to 1.2

## 1.2 Sensors initialisation

Actions :

- Check communication with sensors

- Calibrate all sensors (if needed)

- Check that values measured are correct (TBD: Nominal value ranges)

Exit:

- Once everything is ready and all the tests are OK -> Go to 1.3

- IF a test fails, go to 1.4

## 1.3 Ready for launch

No specific actions

Wait for launch

Exit: When the take off is detected -> Go to 2.1

Take off detection condition:

- First possibility: Umbilical unplugged

- Second possibility: Umbilical unplugged OR [speed AND acceleration > 0 for a certain time t (t TBD)] 						or rather > ε (ε TBD)

The choice will be made later based on how much fluctuations there is on the speed and acceleration at rest.

## 1.4 Sensor error

Enter: One or several sensors are not responding OR are returning absurd values

Action:

- Send an error message through the telemetry

- Stop everything

The system can still be restarted using a reset command

# Ascent

## 2.1 Powered Ascent

No specific actions

Strictly forbid the ejection of the parachute in this phase.

Exit:

At anytime, if the trajectory leaves the security cone -> go to 2.1.1

When one of these conditions is true -> go to 2.2:

- t > combustion time

- Engine off signal from the ground

- (Engine turned off by the engine module itself)	Complicated and not necessarily useful

- (Engine turned off by the trajectory module because the trajectory is too degraded)	Shouldn’t happen since we should have already moved to 2.1.1

TODO: Consider the case where the engine doesn’t work as expected (Mostly, the thrust isn’t nominal)

## 2.1.1 Degraded trajectory

Enter: The trajectory leaves the security cone

Actions: Turn off the engine and then go to 2.2.1

## 2.2 Coasting Ascent

No specific actions

Exit: When the apogee is detected -> Go to 3.1

Apogee detection: The apogee can be detected by the sensors, but only inside a security time window [time_apogee_simulated – t sec, time_apogee_simulated + t sec] (With t TBD). If the apogee hasn’t been detected at the end of the window, the parachute is opened anyway.

Detection by sensors:

- First possibility: Vertical speed < ε (TBD)

- Second possibility: dP/dt >= 0

Summary: Exit when [apogee detected AND current time > time_apogee_simulated – t sec] OR [current time > time_apogee_simulated + t sec]

## 2.2.1 Degraded trajectory

Enter: The trajectory leaves the security cone

Exit: When the apogee is detected -> Go to 3.1

Here, the apogee detection relies only on sensors, the security window is ignored

# Descent (incomplete)

## 3.1 Fast descent

Deploy the 1st parachute

Exit : t > stabilisation time (TBD)

## 3.1.1 Nominal fast descent

Enter : IF Vz < Vmax (TBD)

No specific actions

Exit : z < z0 (TBD)

Exit action : Deploy the 2nd parachute

## 3.1.2 1st parachute failure

Enter : IF Vz > Vmax (TBD)

Actions : Deploy the 2nd parachute

Exit : Immediately

## 3.2 Slow descent

No specific actions

Exit : t > stabilisation time 2 (TBD)

## 3.2.1 Nominal slow descent

Enter : IF Vz < Vmax 2 (TBD)

No specific actions

Exit : Landing <TODO : How to detect ?>

## 3.2.2 2nd parachute failure

Enter : IF Vz > Vmax 2 (TBD)

# Background Tasks

## Data collection and filtering

- Collect data from the sensors

- Check that the values are believable

- Log the raw values from sensors

- Kalman filtering

- Update the position and speed with new data (And keep in mind the time at which these data were measured)

- Log the results

## Trajectory simulation

- Launch a simulation with the current initial conditions (TBD: How often?)

- Log results (TBD: What exactly?)

- …

## Thrust modulation

Currently working on it…

## Trajectory verification

- Define a nominal and a security cone: If the rocket goes outside the nominal cone the trajectory is degraded (for now, nothing is done though), but if ot goes out of the security cone the engine can be turned off

TODO: Nominal & Security envelope => Ask stephanie (+ Check scalar)

## Logs

- Log measures (see 1.)

- Log major events <TODO : what ?>

- Send relevant data to the ground through the telemetry <TODO: What ?>

## Commands from the ground

Execute commands coming from the ground, such as :

- Reset button

- Turn off the engine

- Deploy the parachute

<TODO : Consider the safety of this (require a password ?)>